Rev.: 56086
Last modified: 2019-03-11

P/N: 78506-0201

FLIR E85 24° + 14° + 42°

- zadavatel specifikuje, které dva ze tří objektivů požaduje

| Imaging and optical data |  |
| --- | --- |
| Infrared resolution | 384 × 288 pixels |
| UltraMax (super-resolution) | In FLIR Tools |
| NETD | • <30 mK, 42° @ +30°C (+86°F) • <40 mK, 24° @ +30°C (+86°F) • <50 mK, 14° @ +30°C (+86°F) |
| Field of view | • 42° × 32° • 24° × 18° • 14° × 10° |
| Minimum focus distance | • 0.15 m (0.49 ft.), 42° • 0.15 m (0.49 ft.), 24° • 1.0 m (3.28 ft.), 14° |
| Minimum focus distance with MSX | • 0.65 m (2.13 ft.), 42° • 0.5 m (1.64 ft.), 24° • 1.0 m (3.28 ft.), 14° |
| Focal length | • 10 mm (0.39 in.), 42° • 17 mm (0.67 in.), 24° • 29 mm (1.41 in.), 14° |
| Spatial resolution (IFOV) | • 2.00 mrad/pixel, 42° • 1.09 mrad/pixel, 24° • 0.63 mrad/pixel, 14° |
| Lens identification | Automatic |
| f number | • 1.1, 42° • 1.3, 24° • 1.5, 14° |
| Image frequency | 30 Hz |
| Focus | • Continuous LDM • One-shot LDM • One-shot contrast • Manual |
| Field of view match | Yes |
| Digital zoom | 1–4× continuous |

| Detector data |  |
| --- | --- |
| Focal plane array/spectral range | Uncooled microbolometer/7.5–14 µm |
| Detector pitch | 17 µm |

| Image presentation |  |
| --- | --- |
| Resolution | 640 × 480 pixels (VGA) |
| Surface brightness (cd/m2) | 400 |
| Screen size | 4 in. |
| Viewing angle | 80° |
| Color depth (bits) | 24 |
| Aspect ratio | 4:3 |
| Auto-rotation | Yes |
| Touchscreen | Optically bonded PCAP |
| Display technology | IPS |
| Cover glass material | Dragontrail® |
| Programmable buttons | 1 |
| Viewfinder | No |
| Image adjustment | • Automatic • Automatic maximum • Automatic minimum • Manual |

| Image presentation modes |  |
| --- | --- |
| Infrared image | Yes |
| Visual image | Yes |
| Thermal fusion | No |
| MSX | Yes |
| Picture in Picture | Resizable and movable |
| Gallery | Yes |

| Measurement |  |  |
| --- | --- | --- |
| Camera temperature range | Object temperature range | Accuracy — for ambient temperature +15 to +35°C (+59 to +95°F) |
| –20 to +120°C (–4 to +248°F) | –20 to +100°C (–4 to +212°F) | ±2°C (±3.6°F) |
|  | +100 to +120°C (+212 to +248°F) | ±2% |
| 0 to +650°C (+32 to +1202°F) | 0 to +100°C (+32 to +212°F) | ±2°C (±3.6°F) |
|  | +100 to + 650°C (+212 to +1202°F) | ±2% |
| +300 to +1200°C (+572 to +2192°F) | +300 to +1200°C (+572 to +2192°F) | ±2% |

| Measurement analysis |  |
| --- | --- |
| Spotmeter | 3 in live mode |
| Area | 3 in live mode |
| Automatic hot/cold detection | Auto-maximum/minimum markers within area |
| Measurement presets | • No measurements • Center spot • Hot spot • Cold spot • User preset 1 • User preset 2 |
| Difference temperature | Yes |
| Reference temperature | Yes |
| Emissivity correction | Yes: variable from 0.01 to 1.0 or selected from materials list |
| Measurement corrections | Yes |
| External optics/windows correction | Yes |
| Screening | 0.5°C (0.9°F) accuracy @ 37°C (98.6°F) with reference |

| Alarm |  |
| --- | --- |
| Color alarm (isotherm) | • Above • Below • Interval • Condensation (moisture/humidity/dewpoint) • Insulation |
| Measurement function alarm | Audible/visual alarms (above/below) on any selected measurement function |

| Set-up |  |
| --- | --- |
| Color palettes | • Iron • Gray • Rainbow • Arctic • Lava • Rainbow HC |
| Setup commands | Local adaptation of units, language, date and time formats |
| Languages | 21 |

| Service functions |  |
| --- | --- |
| Camera software update | Use PC software FLIR Tools |

| Storage of images |  |
| --- | --- |
| Storage media | Removable memory; SD card (8 GB) |
| Time lapse (periodic image storage) | No |
| Remote control operation | • Using FLIR Tools (using USB cable) • FLIR Tools Mobile (over Wi-Fi) |
| Image file format | Standard JPEG, measurement data included. Infrared-only mode |

| Image annotations |  |
| --- | --- |
| Voice | 60 seconds built-in microphone and speaker (and via Bluetooth) on still images and video |
| Text | Text from predefined list or soft keyboard on touchscreen |
| Visual image annotation | Yes |
| Image sketch | Yes: on infrared images only |
| Sketch | From touchscreen |
| METERLiNK | Wireless connection (Bluetooth) to: FLIR meters with METERLiNK |
| Compass | Yes |
| Laser distance meter information | Yes |
| Area measurement information | Yes |
| GPS | Yes: location data automatically added to every still image and the first frame in video from built-in GPS |

| Video recording in camera |  |
| --- | --- |
| Radiometric infrared-video recording | RTRR (.csq) |
| Non-radiometric infrared-video recording | H.264 to memory card |
| Visual video recording | H.264 to memory card |

| Video streaming |  |
| --- | --- |
| Radiometric infrared–video streaming (compressed) | Over UVC |
| Non-radiometric video streaming (compressed: IR, MSX, visual, Picture in Picture) | • H.264 (AVC) over RTSP (Wi-Fi) • MPEG4 over RTSP (Wi-Fi) • MJPEG over UVC and RTSP (Wi-Fi) |
| Visual video streaming | Yes |

| Digital camera |  |
| --- | --- |
| Resolution | 5 MP with LED light |
| Focus | Fixed |
| Field of view | 53° × 41° |
| Video lamp | Built-in LED light |

| Laser pointer |  |
| --- | --- |
| Laser alignment | Position is automatically displayed on the infrared image |
| Laser distance meter | Activated by a dedicated button |
| Laser | Class 2, 0.05–40 m (1.6–131 ft.) ±1% of measured distance |

| Data communication interfaces |  |
| --- | --- |
| Interfaces | USB 2.0, Bluetooth, Wi-Fi, DisplayPort |
| METERLiNK/Bluetooth | Communication with headset and external sensors |
| Wi-Fi | Peer to peer (ad hoc) or infrastructure (network) |
| Audio | Microphone and speaker for voice annotation of images |
| USB | USB Type-C: data transfer/video/power |
| USB standard | USB 2.0 High Speed |
| Video out | DisplayPort |
| Video connector type | DisplayPort over USB Type-C |

| Radio |  |
| --- | --- |
| Operating frequency | Bluetooth + EDR/LE: 2402–2480 MHz WLAN 2.4 GHz: 2412–2462 MHz WLAN 5 GHz: 5150–5350 MHz (DFS: only slave mode) Note that frequency band 5150–5350 MHz is for indoor use only, see national regulations. |
| RF output (EIRP) | Bluetooth + EDR/LE: < 10 dBm WLAN: < 17 dBm |
| Antenna | Integrated PIFA antenna (gain: maximum 1.4 dBi) |

| Power system |  |
| --- | --- |
| Battery type | Rechargeable Li-ion battery |
| Battery voltage | 3.6 V |
| Battery operating time | > 2.5 hours at 25°C (68°F) and typical use |
| Charging system | In camera (AC adapter or 12 V from a vehicle) or two-bay charger |
| Charging time (using two-bay charger) | 2.5 hours to 90% capacity with charging status indicated by LEDs |
| Charging temperature | 0°C to +‎45°C (+‎32°F to +‎113°F)‎, except for the Korean market: +10°C to +‎45°C (+‎50°F to +‎113°F)‎ |
| External power operation | AC adapter 90–260 V AC, 50/60 Hz, or 12 V from a vehicle (cable with standard plug—optional) |
| Power management | Automatic shut-down and sleep mode |

| Environmental data |  |
| --- | --- |
| Operating temperature range | –15 to +50°C (5–122°F) |
| Storage temperature range | –40 to +70°C (–40 to +158°F) |
| Humidity (operating and storage) | IEC 60068-2-30/24 hours/95% relative humidity 25–40°C (77–104°F)/two cycles |
| EMC | • ETSI EN 301 489-1 (radio) • ETSI EN 301 489-17 • EN 61000-6-2 (immunity) • EN 61000-6-3 (emission) • FCC 47 CFR Part 15 Class B (emission) |
| Radio spectrum | • ETSI EN 300 328 • FCC Part 15.249 • RSS-247 Issue 2 |
| Encapsulation | IP 54 (IEC 60529) |
| Shock | 25g (IEC 60068-2-27) |
| Vibration | 2g (IEC 60068-2-6) |
| Drop | Designed for 2 m (6.6 ft.) |
| Safety | EN/UL/CSA/PSE 60950-1 |

| Physical data |  |
| --- | --- |
| Weight (including battery) | 1 kg (2.2 lb.) |
| Size (L × W × H) | 278.4 × 116.1 × 113.1 mm (11.0 × 4.6 × 4.4 in.) |
| Battery weight | 140 g (4.9 oz.) |
| Battery size (L × W × H) | 150 × 46 × 55 mm (5.9 × 1.8 × 2.2 in.) |
| Tripod mounting | UNC ¼″-20 |
| Housing material | PCABS with TPE, magnesium |
| Color | Black |

| Warranty and service |  |
| --- | --- |
| Warranty |  |

| Shipping information |  |
| --- | --- |
| Packaging, type | Cardboard box |
| Packaging, contents | • Extra lens, 14°‎ • Extra lens, 42°‎ • Battery (2 ea)‎ • Battery charger • Hard transport case • Infrared camera with lens • Lens cap,‎ front • Lens cap,‎ front and rear (only for extra lenses) • Accessory Box I: ◦ SD card (8 GB)‎ ◦ USB 2.0 A to USB Type-C cable,‎ 1.0 m ◦ USB Type-C to HDMI adapter,‎ standard specification UH311 ◦ USB Type-C to USB Type-C cable (USB 2.0 standard)‎,‎ 1.0 m ◦ Power supply for battery charger ◦ Power supply,‎ 15 W/3 A ◦ Printed documentation • Accessory box II: ◦ Hand strap ◦ Lanyard strap,‎ camera ◦ Wrist strap ◦ Lens cap strap ◦ Front protection ◦ Carabiner hook ◦ Accessory box III: – Hand strap bracket, right – Hand strap bracket, left – Torx T10 wrench – Front protection fastener – Screws |
| Packaging, weight | 6.4 kg (14.1 lb.) |
| Packaging, size | 500 × 190 × 370 mm (19.7 × 7.5 × 14.6 in.) |
| EAN-13 | 4743254002791 |
| UPC-12 | 845188014049 |
| Country of origin | Estonia |