Příloha č. 2 Kupní smlouvy

Annex 2   to the Purchase Agreement

# Strain-gauge data acquisition system for CTU FTS

Elsys AG Solution Description

## Strain Gauge Pre-Amplifier

| Position | # of Pcs | Specification |
| --- | --- | --- |
| 20 | 1 | Amp-BU-24-MK2 - 19 Inch Rack - Up to 12 SGA-MK2 Rack Insert Modules (24 Channels) - 230V Power Supply - USB Interface for channel configuration - Incl. Control Software |
| 21 | 6 | SGA2-MK2 Strain Gauge Amplifier - Gain 1, 10 , 100 - 5V / 10 V Bridge Excitation - Quarter/ Half / Full Bridge Mode - 7-Pol Lemo Input Connector, BNC Output - Auto-Bridge Balancing - High Band-Width (1.5MHz at Gain 1/10) - https://www.elsys-instruments.com/en/products/straingaugeamplifier.php |
| 22 | 12 | Lemo cable connector 7-pol - FGG.1B.307.CLAD62 |
| 23 | 12 | BNC Cable SGA2-MK2 DAQ |

Picture as example only

## Data Acquisition System

| Position | # of Pcs | Specification |
| --- | --- | --- |
| 10 | 1 | TraNET EPC - Industrial PC in a 19 Inch case for up to 32 channels - Intel i5 or better CPU, 8 GB RAM, SSD System drive - Windows 10, 64 Bit - 1 TB Data Drive - without Monitor - https://www.elsys-instruments.com/en/products/data-acquisition-system-tranet-epc.php |
| 11 | 1 | TPCE-1016-8S DAQ Module - 8-ch Single Ended or 4-ch differential inputs - Max Sampling rate: 5 MS/s 16 Bit, 10 MS/s 14 Bit - On-Board Memory 8x16M - Input Range per channel: ±50mV - ±50V - Anti-Aliasing Filter, 200 Hz – 5 MHz cut-off frequency - BNC Input Connector - https://www.elsys-instruments.com/en/products/tpce_data_acquisition_card_pcie.php |
| 12 | 1 | TPCE-1016-4D DAQ Module - 4-ch Single Ended or 4-ch differential inputs - Max Sampling rate: 5 MS/s 16 Bit, 10 MS/s 14 Bit - On-Board Memory 4x32M - Input Range per channel: ±50mV - ±50V - Anti-Aliasing Filter, 200 Hz – 5 MHz cut-off frequency - BNC Input Connector (+- BNC-Pair per Input Channel) - https://www.elsys-instruments.com/en/products/tpce_data_acquisition_card_pcie.php |
| 30 | 1 | TranAX 4 Data Acquisition Software - https://www.elsys-instruments.com/en/products/tranax4_data_acquisition_software.php |

TraNET EPC – Example Image

TranAX 4 DAQ Software

## Additional Services included in the price

- Calibration Certificate for SGA Amplifier and DAQ Modules

- ½ Day Online Training

- Transport and Insurance

- 24 Month Warranty

Niederrohrdorf, 05.06.2020/RB