| Confidentiality Level |  | BL - Restricted for internal use | TC ID / Revision |  | 00148510/C |
| --- | --- | --- | --- | --- | --- |
| Document Status |  | Document Released | Document No. |  | N/A |
| WBS code |  | 4.4 - Control System, 1.9 - ICT |  |  |  |
| PBS code |  | SE.BDS.CS.HW.4.40, MGNT.ITS.1 |  |  |  |
| Project branch |  | Engineering & Scientific documents (E&S) |  |  |  |
| Document Type |  | Specification (SP) |  |  |  |
| [RSD product category A] Operators computers TP15_029e Keywords N/A |  |  |  |  |  |
|  | Position |  |  | Name |  |
| Responsible person | CS Group Leader |  |  | Pavel Bastl |  |
| Prepared by | Software engineer |  |  | Ondřej Janda |  |

| RSS TC ID/revision | RSS - Date of Creation | RSS - Date of Last Modification | Systems Engineer |
| --- | --- | --- | --- |
| 011947/A.001 | 10.04.2017 19:22 | 10.04.2017 19:23 | Aleksei Kuzmenko |
| 011947/A.002 | 11.04.2017 16:42 | 11.04.2017 16:43 | Aleksei Kuzmenko |
| 011947/A.003 | 26.04.2017 10:47 | 26.04.2017 10:47 | Aleksei Kuzmenko |

| Reviewed By |  |  |  |
| --- | --- | --- | --- |
| Name (Reviewer) | Position | Date | Signature |
| Jaroslav Charfreitag | IT Leader | NOTICE (RSD product category A) |  |
| Ladislav Půst | Manager installation of technology | NOTICE (RSD product category A) |  |
| Luboš Nims | Head of Electrical engineering | NOTICE (RSD product category A) |  |
| Pavel Bastl | CS Group Leader | NOTICE (RSD product category A) |  |
| Petr Pivoňka | Control System Engineer | NOTICE (RSD product category A) |  |
| Veronika Olšovcová | Safety Coordinator | NOTICE (RSD product category A) |  |
| Viktor Fedosov | SE & Planning group leader; Quality Manager | NOTICE (RSD product category A) |  |

| Approved by |  |  |  |
| --- | --- | --- | --- |
| Name (Approver) | Position | Date | Signature |
| Georg Korn | Science and Technology Manager, Scientific coordinator of RP2-6 |  |  |
| Johana Burgetová | Administrative Manager, Head of Department 96, Financial Group Leader |  |  |

| Revision History / Change Log |  |  |  |  |
| --- | --- | --- | --- | --- |
| Change No. | Made by | Date | Change description, Pages, Chapters | TC rev. |
| 1 | A.Kuzmenko | 10.04.2017 | RSD draft creation | A |
| 2 | A.Kuzmenko | 11.04.2017 | RSD update, standard chapters have been added, version for review | B |
| 3 | A.Kuzmenko | 26.04.2017 | RSD update, final version for approval | C |

Table of Content

1. Introduction	4

1.1. Purpose	4

1.2. Scope	4

1.3. Terms, Definitions and Abbreviations	4

2. Functional, Performance and Design requirements	5

2.1. Solid-State Drive (SSD)	5

2.2. Hard Disk Drive (HDD)	6

2.3. Graphics Processing Unit (GPU)	7

2.4. Random-Access Memory (RAM)	8

2.5. Motherboard	8

2.6. Power Supply Unit (PSU)	10

2.7. Mouse	12

2.8. Keyboard	12

3. Delivery Requirements	13

4. Safety Requirements	13

5. Quality Requirements	14

5.1. General Quality Requirements	14

# Introduction

## Purpose

This Requirements Specification Document (RSD) lists the technical requirements and constraints on product applying in Control Systems of ELI project. This can lead to the identification of product interfaces with the ELI science based technology and ELI building facility. This RSD also acts as the parent document for the technical requirements that need to be addressed in lower level design description documents.

## Scope

This RSD contains all of the technical requirements: functional, performance and design, delivery, safety and quality requirements for the following product: Operators computers (PBS: SE.BDS.CS.HW.4.40, MGNT.ITS.1). These products are products Category A.

The Category A is an Off-the-shelf Product without necessity of modifications and necessity to be subjected to a verification program (review of design, inspection and testing) for ELI applications by the actual project specifications. All verification activities performing by a supplier shall be executed in accordance with the supplier’s plan of outgoing inspection and tests. Internal Acceptance Procedure of the product Category A shall be established and applied before the product implementation (operation phase).

## Terms, Definitions and Abbreviations

For the purpose of this document, the following abbreviations apply:

| Abbreviation | Meaning |
| --- | --- |
| AES | Advanced Encryption Standard |
| COM port | Communication port |
| DDR4 | Double Data Rate 4 |
| ECC memory | Error-Correcting Code memory |
| ELI | Extreme Light Infrastructure |
| GDDR5 | Graphic Double Data Rate 5 |
| GPU | Graphics Processing Unit |
| HDD | Hard Disk Drive |
| ID | Identification |
| IOPS | Input/output Operations Per Second |
| IPMI v2 | Intelligent Platform Management Interface |
| LAN | Local Area Network |
| NAS | Network-Attached Storage |
| NVMe | Non-Volatile Memory Express |
| PCI-E | Peripheral Component Interconnect Express |
| PFC | Power Factor Correction |
| PSU | Power Supply Unit |
| RAID | Redundant Array of Independent Disks |
| RAM | Random-Access Memory |
| RPM | Revolutions Per Minute |
| RSD | Requirements Specification Document |
| RSTe | Rapid Storage Technology enterprise |
| RTC | Real-Time Clock |
| SATA | Serial Advanced Technology Attachment |
| SDRAM | Synchronous Dynamic Random Access Memory |
| SFP+ | Small Form-factor Pluggable |
| SMBIOS | System Management BIOS |
| SPI | Serial Peripheral Interface |
| SSD | Solid-State Drive |
| TCP | Transmission Control Protocol |
| TLC | Triple-Level Cells |
| TPM | Trusted Platform Module |
| UEFI | Unified Extensible Firmware Interface |
| USB | Universal Serial Bus |
| VGA | Video Graphics Array |

# Functional, Performance and Design requirements

## Solid-State Drive (SSD)

REQ-020674/A

The parameters of all the SSDs for PC shall correspond to the requirements given in the Table 1 – “Functional and performance parameters of the SSDs for PC”.

| Item N° | Parameter name | Parameter description |
| --- | --- | --- |
| 1 | GENERAL |  |
| 1.1 | Dimensions | 22 mm (w) x 1.5 mm (h) x 80 mm (d) |
| 1.2 | Interface | PCI-E 3.0 x4 NVMe |
| 1.3 | Form Factor | M.2 2280 |
| 2 | Capacity |  |
| 2.1 | Capacity Size | 256 GB |
| 2.2 | Memory Components | TLC |
| 3 | Performance |  |
| 3.1 | Sequential Read | Up to 1570 MB/sec |
| 3.2 | Sequential Write | Up to 540 MB/sec |
| 3.3 | Random Read 4KB | 71000 IOPS |
| 3.4 | Random Write 4KB | 112000 IOPS |
| 4 | Features |  |
| 4.1 | Reliability (MTBF) | 1.6 Million Hours |
| 4.2 | Encryption Support | AES 256-bit |

Table 1: Functional and performance parameters of the SSDs for PC.

## Hard Disk Drive (HDD)

REQ-020675/A

The parameters of all the HDDs for NAS or Servers shall correspond to the requirements given in the Table 2 – “Functional and performance parameters of the HDDs for NAS or Servers”.

| Item N° | Parameter name | Parameter description |
| --- | --- | --- |
| 1 | GENERAL |  |
| 1.1 | Dimensions | 101,85 x 26,11 x 146,99 mm (3,5" standard) |
| 1.2 | Interface | SATA III (SATA 6 Gb/s) |
| 1.3 | Form Factor | 3,5" slot |
| 1.4 | Usage | Servers, Data Storage (not for workstations or PC), 24/7 |
| 2 | Capacity |  |
| 2.1 | Capacity Size | 10 TB (10 000 GB) |
| 2.2 | Rotate | min. 7200 RPM |
| 2.3 | Cache | min. 256 MB |
| 3 | Performance and Power |  |
| 3.1 | Maximum power consumption | max. 7 W |
| 3.2 | Standby consumption | max. 1 W |
| 3.3 | Average Read | min. 180 MB/s |
| 3.4 | 4K Block Size READ | min. 85 MB/s |
| 3.5 | 4K Block Size WRITE | min. 90 MB/s |
| 4 | Features |  |
| 4.1 | Reliability (MTBF) | 1.5 Million Hours |
| 4.2 | RAID Support | YES |

Table 2: Functional and performance parameters of the HDDs for NAS or Servers.

## Graphics Processing Unit (GPU)

REQ-020676/A

The parameters of all the GPUs shall correspond to the requirements given in the Table 3 – “Functional and performance parameters of the GPUs”.

| Item N° | Parameter name | Parameter description |
| --- | --- | --- |
| 1 | GENERAL |  |
| 1.1 | Dimensions | 229 mm (w) x 118 mm (h) x 40 mm (d) |
| 1.2 | Interface | PCI-E 3.0 x16 |
| 1.3 | Slot Width | Dual Slot |
| 2 | GPU |  |
| 2.1 | CUDA Cores | 768 |
| 2.2 | Core Clock | 1354 MHz |
| 2.3 | Boost Clock | 1468 MHz |
| 3 | Memory |  |
| 3.1 | Memory Type | GDDR5 |
| 3.2 | Memory Size | 4 GB |
| 3.3 | Memory Interface | 128 bit |
| 3.4 | Memory Effective Clock | 7008 MHz |
| 4 | Ports |  |
| 4.1 | Native HDMI 2.0b | 3 x |
| 4.2 | Native DP 1.4 | 1 x |
| 4.3 | Native DVI-D | 1 x |
| 5 | Power Connectors and Cooling |  |
| 5.1 | Cooling Type | Active |
| 5.2 | Fan Size | Two Slots |
| 5.3 | Power Connector | 6-Pin |
| 6 | Features |  |
| 6.1 | Max Resolution | 7680 x 4320 @ 60 Hz |
| 6.2 | OpenGL | 4.5 |
| 6.3 | DirectX | 12 |
| 6.4 | OS Compatibility | Linux 64bit and Windows 10 64bit |

Table 3: Functional and performance parameters of the GPUs.

## Random-Access Memory (RAM)

REQ-020677/A

The parameters of all the RAMs shall correspond to the requirements given in the Table 4 – “Functional and performance parameters of the RAMs”.

| Item N° | Parameter name | Parameter description |
| --- | --- | --- |
| 1 | GENERAL |  |
| 1.1 | Type | DDR4 SDRAM 288-Pin |
| 1.2 | Modules | 1 |
| 2 | Memory |  |
| 2.1 | Capacity | 16 GB |
| 2.2 | Speed | DDR4 2133 |
| 2.3 | Cas Latency | 14 |
| 3 | Features |  |
| 3.1 | Heat Spreader | Yes |
| 3.2 | Intel XMP-ready | Yes |

Table 4: Functional and performance parameters of the RAMs.

## Motherboard

REQ-020678/A

The parameters of all the motherboards shall correspond to the requirements given in the Table 5 – “Functional and performance parameters of the motherboards”.

| Item N° | Parameter name | Parameter description |
| --- | --- | --- |
| 1 | GENERAL |  |
| 1.1 | Form Factor | Mini-ITX |
| 1.2 | Dimensions | 17cm x 17cm |
| 2 | Processor |  |
| 2.1 | Integrated on the board | Yes |
| 2.2 | Cores | 8 |
| 2.3 | Threads | 16 |
| 2.4 | Processor Base Frequency | At minimum 1.70 GHz |
| 2.5 | CPU TDP | 35 W |
| 2.6 | Cache | 12 MB |
| 3 | System Memory Support |  |
| 3.1 | Number of DDR4 DIMM sockets | 4 |
| 3.2 | Max amount of ECC RDMI memory | 128 GB |
| 3.3 | Max amount of ECC/non ECC UDIM memory | 64 GB |
| 3.4 | DIMM sizes | 4 GB, 8 GB, 16 GB and 32 GB |
| 3.5 | Memory type | 2133/1866/1600 MHz DDR4 |
| 3.6 | Error correction | single bit and double bit (ECC memory) |
| 4 | On-Board Devices (controllers) |  |
| 4.1 | SATA SoC | 6 x SATA3 ports; RAID 0,1,5,10 RSTe |
| 4.2 | Network SoC | 2 x 1GbE (i350-AM2), 2 x 10G SFP+ LAN, 1 x dedicated IPMI v2 |
| 4.3 | Video | Aspeed AST2400 |
| 4.4 | TPM | TPM Header |
| 5 | Input / Output |  |
| 5.1 | SATA | 6 x SATA3 (6Gbps) port |
| 5.2 | 1 GbE LAN | 2 x 1 GbE RJ45 LAN port |
| 5.3 | 10 G LAN | 2 x 10 G SFP+ LAN port and 2 x compatible 10 G LC SM 1310 SFP+ module, SFP+ compatible modules are part of the delivery |
| 5.4 | USB | 2 x USB 3.0 (rear) port, 2 x USB 2.0 (header) port |
| 5.5 | Video | 1 x VGA port |
| 5.6 | Serial Port | 1 x COM port |
| 5.7 | TPM | 1 x TPM Header |
| 6 | Expansion Slots |  |
| 6.1 | PCI-Express | 1 x PCI-E 3.0 x16 slot |
| 6.2 | M.2 | Interface PCI-E 3.0 x4; Form Factor: M Key 2242/2280; SATA devices supported |
| 7 | BIOS |  |
| 7.1 | Type | AMI Bios; 128 MB SPI Flash |
| 7.2 | Features | PCI 3.0; ASPI 5.0; USB Keyboard; SMBIOS 2.8; UEFI 2.4; RTC Wakeup |
| 8 | Management |  |
| 8.1 | OS Compatibility 64b | Red Hat Linux 7.2, CentOS 7.2, Windows 7 SP1, Windows 10 |
| 8.2 | ACPI / APM Power Management | Yes |
| 8.3 | Power-on mode for AC power recovery | Yes |
| 9 | PC Health Monitoring |  |
| 9.1 | RoHS | Yes |
| 9.2 | Thermal Monitor 2 (TM2) support | Yes |
| 9.3 | Monitoring for CPU and chassis environment | Yes |
| 9.4 | 4-pin fan headers supporting tachometer status monitoring | 4 x |
| 9.5 | FAN status monitor for speed control | Yes |
| 9.6 | FAN status monitor for on/off control | Yes |
| 9.7 | Pulse Width Modulated (PWM) fan connectors | Yes |
| 9.8 | Monitors for CPU Cores, +12V, +3.3V, +5V, 1.2V (VDIMM), Chassis intrusion header, System level control | Yes |

Table 5: Functional and performance parameters of the motherboards.

## Power Supply Unit (PSU)

REQ-020679/A

The parameters of all the PSUs shall correspond to the requirements given in the Table 6 – “Functional and performance parameters of the PSUs”.

| Item N° | Parameter name | Parameter description |
| --- | --- | --- |
| 1 | GENERAL |  |
| 1.1 | Dimensions | 100 mm (l) x 125 mm (w) x 63.5 mm (d) |
| 1.2 | Form Factor | SFX 12V (v3.3) |
| 1.3 | Sleeved cables | Yes |
| 2 | Power |  |
| 2.1 | Continuous power | 300 W |
| 2.2 | Peak power | 350 W |
| 2.3 | Voltage | 100 V - 240 V |
| 2.4 | Frequency | 50 Hz - 60 Hz |
| 2.5 | Input current | 4.5 A / 2.5 A |
| 2.6 | PFC | active |
| 2.7 | Power factor at 100% load | >0.97 |
| 3 | Cables and Connectors |  |
| 3.1 | No. of cables total | 6 |
| 3.2 | ATX-Mainboard (20+4 pin) | 1 |
| 3.3 | P4 (CPU) | 1 |
| 3.4 | PCI-E 6+2 pin (GPU) | 1 |
| 3.5 | PATA 4 pin | 1 |
| 3.6 | SATA 15-pin (HDD, Drives) | 4 |
| 3.7 | FDD (Floppy) | 1 |
| 4 | Output |  |
| 4.1 | 12V rails | 2 |
| 4.2 | +3.3V | 21 A |
| 4.3 | +5V | 20 A |
| 4.4 | +12V1 | 14 A |
| 4.5 | +12V2 | 16 A |
| 4.6 | -12V | 0.5 A |
| 4.7 | +5Vsb | 2.5 A |
| 4.8 | Max combined power 12V | 264 W |
| 4.9 | Max combined power 3.3V + 5V | 103 W |
| 5 | Cooling |  |
| 5.1 | Fan size | 80 mm |
| 5.2 | Max fan speed | 2219 rpm |
| 5.3 | Sound pressure levels at 20% load | 16.6 dB(A) |
| 5.4 | Sound pressure levels at 50% load | 17.2 dB(A) |
| 5.5 | Sound pressure levels at 100% load | 31.4 dB(A) |
| 6 | Efficiency |  |
| 6.1 | 80PLUS certification | Bronze |
| 6.2 | Efficiency at 230V, 20% load | At least 87% |
| 6.3 | Efficiency at 230V, 50% load | At least 89% |
| 6.4 | Efficiency at 230V, 100% load | At least 86% |
| 6.5 | Energy Star | 6.0 |

Table 6: Functional and performance parameters of the PSUs.

## Mouse

REQ-020680/A

The parameters of all the mouse devices shall correspond to the requirements given in the Table 7 – “Functional and performance parameters of the mouse devices”.

| Item N° | Parameter name | Parameter description |
| --- | --- | --- |
| 1 | GENERAL |  |
| 1.1 | Type | Laser mouse |
| 1.2 | Design (left, right hand) | Symetric |
| 1.3 | Color | Orange |
| 1.4 | Interface | USB 2.0 - wired |
| 1.5 | Rubberized surface | Yes |
| 2 | Performance |  |
| 2.1 | DPI | Up to 8200 |
| 2.2 | Rate | Up to 1000 Hz |
| 2.3 | Min Response | 2 ms |
| 3 | Features |  |
| 3.1 | Number of programable buttons | 9 |
| 3.2 | Wheel | Yes |

Table 7: Functional and performance parameters of the mouse devices.

## Keyboard

REQ-020681/A

The parameters of all the keyboards shall correspond to the requirements given in the Table 8 – “Functional and performance parameters of the keyboards”.

| Item N° | Parameter name | Parameter description |
| --- | --- | --- |
| 1 | GENERAL |  |
| 1.1 | Localisation | Czech |
| 1.2 | Element arrangement | Classic |
| 1.3 | Key type | Flat keys with scissor switch |
| 1.4 | Interface | USB - wired |
| 2 | Keys |  |
| 2.1 | Left shift | Wide |
| 2.2 | Cursor arrow | Classic |
| 2.3 | Enter | Two-line narrow |
| 2.4 | Backspace | Wide |
| 3 | Features |  |
| 3.1 | Backlit keys | Yes up to 7 colors |
| 3.2 | Multimedia keys | Yes |
| 3.3 | Anti-Ghosting | Yes |
| 3.4 | Rubber Stands | Yes |

Table 8: Functional and performance parameters of the keyboards.

# Delivery Requirements

REQ-020682/A

The transportation to the final destination shall be conducted by the Supplier.

REQ-020683/A

All devices shall be delivered in protective package preventing damage.

# Safety Requirements

REQ-020684/A

The Supplier shall supply a Declaration of Conformity or any other equivalent document legally recognized and accepted in the Czech Republic for each product type if the appropriate legislation determines the Supplier's obligation to have a Declaration of Conformity (or the equivalent document) for the purposes of a Device sale in the Czech Republic to fulfil the requirements of 2001/95/EC directive or applicable Czech law.

# Quality Requirements

## General Quality Requirements

REQ-020685/A

The Supplier shall provide the Product User Manual as part of the delivered Device. The Manual shall include the instructions and descriptions regarding the following procedures:

- transport, handling, storage and installation;

- safe operation and maintenance procedures.

REQ-020686/A

The Supplier shall provide information on execution of outgoing check of the Product. At least this information shall comprise declaration about execution of outgoing check and declaration of conformity with technical requirements defined by the product RSD and completeness of the Product.

NOTE: Alternatively the Supplier might provide the Contracting Authority with information (e.g.: catalogue/technical data sheets, product manuals or other similar documentation) subject to the condition that such documentation is detailed enough to prove meeting all requirements stipulated herein.

REQ-020687/A

The Supplier shall establish and maintain a non-conformance control system compatible with ČSN EN ISO 9001.